Finance Switch:

en

config t

hostname Finance-SW

enable password cisco

line console 0

password cisco

login

exit

banner motd #N0 UNAUTHORISED ACCESS, THIS IS PUNISHABLE BY LAW!!!#

service password-encryption

no ip domain lookup

do wr

AFTER BASIC CONFIGURATION:

vlan 10

name DATA

vlan 100

name VOICE

int fa0/1

switchport mode trunk

exit

int range fa0/2-24

switchport mode access

switchport access vlan 10

switchport voice vlan 100

do wr

HR Switch:

en

config t

hostname HR-SW

enable password cisco

line console 0

password cisco

login

exit

banner motd #N0 UNAUTHORISED ACCESS, THIS IS PUNISHABLE BY LAW!!!#

service password-encryption

no ip domain lookup

do wr

AFTER BASIC CONFIGURATION:

vlan 20

name DATA

vlan 100

name VOICE

int fa0/1

switchport mode trunk

exit

int range fa0/2-24

switchport mode access

switchport access vlan 20

switchport voice vlan 100

do wr

Sale Switch:

en

config t

hostname Sale-SW

enable password cisco

line console 0

password cisco

login

exit

banner motd #N0 UNAUTHORISED ACCESS, THIS IS PUNISHABLE BY LAW!!!#

service password-encryption

no ip domain lookup

do wr

AFTER BASIC CONFIGURATION:

vlan 30

name DATA

vlan 100

name VOIC

int fa0/1

switchport mode trunk

exit

int range fa0/2-24

switchport mode access

switchport access vlan 30

switchport voice vlan 100

do wr

ICT Switch:

en

config t

hostname ICT-SW

enable password cisco

line console 0

password cisco

login

exit

banner motd #N0 UNAUTHORISED ACCESS, THIS IS PUNISHABLE BY LAW!!!#

service password-encryption

no ip domain lookup

do wr

AFTER BASIC CONFIGURATION:

vlan 40

name DATA

vlan 100

name VOICE

int fa0/1

switchport mode trunk

exit

int range fa0/2-24

switchport mode access

switchport access vlan 40

switchport voice vlan 100

do wr

SERVER SWITCH:

vlan 40

name DATA

int fa0/1

switchport mode trunk

exit

int range fa0/2-5

switchport mode access

switchport access vlan 50

do wr

FINANCE ROUTER:

en

config t

hostname Fin-Router

enable password cisco

line console 0

password cisco

login

exit

banner motd #N0 UNAUTHORISED ACCESS, THIS IS PUNISHABLE BY LAW!!!#

service password-encryption

no ip domain lookup

do wr

username cisco password cisco

ip domain name cisco.net

crypto key generate rsa general-keys modulus 1024

line vty 0 15

login local

transport input ssh

exit

do wr

INTERFACES

CONFIGURE DHCP FOR VOICE:

Inter-VLAN routing on the Routers Plus Ip dhcp helper Address

OSPF on the Routers

Configure Voip Configuration:

Dial Peering Configuration:

HR ROUTER:

en

config t

hostname HR-Router

enable password cisco

line console 0

password cisco

login

exit

banner motd #N0 UNAUTHORISED ACCESS, THIS IS PUNISHABLE BY LAW!!!#

service password-encryption

no ip domain lookup

do wr

username cisco password cisco

ip domain name cisco.net

crypto key generate rsa general-keys modulus 1024

line vty 0 15

login local

transport input ssh

exit

do wr

INTERFACES

CONFIGURE DHCP FOR VOICE:

Inter-VLAN routing on the Routers Plus ip dhcp helper Address

OSPF on the Routers

Configure Voip Configuration:

Dial Peering Configuration:

SALE ROUTER

en

config t

hostname SALE-Router

enable password cisco

line console 0

password cisco

login

exit

banner motd #N0 UNAUTHORISED ACCESS, THIS IS PUNISHABLE BY LAW!!!#

service password-encryption

no ip domain lookup

do wr

username cisco password cisco

ip domain name cisco.net

crypto key generate rsa general-keys modulus 1024

line vty 0 15

login local

transport input ssh

exit

do wr

INTERFACES

CONFIGURE DHCP FOR VOICE:

Inter-VLAN routing on the Routers Plus ip dhcp helper Address

OSPF on the Routers

Configure Voip Configuration:

Dial Peering Configuration:

ICT ROUTER:

en

config t

hostname ICT-Router

enable password cisco

line console 0

password cisco

login

exit

banner motd #N0 UNAUTHORISED ACCESS, THIS IS PUNISHABLE BY LAW!!!#

service password-encryption

no ip domain lookup

do wr

username cisco password cisco

ip domain name cisco.net

crypto key generate rsa general-keys modulus 1024

line vty 0 15

login local

transport input ssh

exit

do wr

INTERFACES

CONFIGURE DHCP FOR VOICE:

Inter-VLAN routing on the Routers Plus ip dhcp helper Address

Inter-VLAN routing on the Routers Plus ip dhcp helper Address For SERVER

OSPF on the Routers

Configure Voip Configuration:

Dial Peering Configuration: